OWNERS MANUAL

MODEL:

METROPOLITAN 4

Professional Popcorn Machine

PART NUMBER:

11045

115 volts, 60 hz

WARNING ELECTRICAL SHOCK HAZARD.

FAILURE TO FOLLOW THESE INSTRUCTIONS COULD RESULT IN SERIOUS INJURY OR DEATH.

WARNING: Handling the cord on this product may expose you to lead, a chemical known to the State Of California to cause cancer and birth defects or other reproductive harm. WASH HANDS AFTER HANDLING.

Electrical ground is required on this appliance.

- Do not modify the power supply cord plug. If it does not fit the outlet, have a proper outlet installed by a qualified electrician.

- Do not use an extension cord with this appliance.

- Check with a qualified electrician if you are in doubt as to whether the appliance is properly grounded.

SAFETY INSTRUCTIONS

1.      Plug electrical cord into a certified electrical outlet with a corresponding voltage rating as listed on the machine model labels.

2.      Always unplug the electric cord when appliance is not in use and before cleaning, adjusting or maintaining this steamer. Do not operate with a damaged cord or plug or after the machine malfunctions, has been dropped or damaged. Use authorized service for examination, electrical or mechanical adjustment or repair.

3       This equipment will be HOT during use and for a short time after it is turned off  BE CAREFUL. To avoid burns, do not let bare skin touch hot surfaces. If provided, use handles when moving this appliance. Keep combustible materials away from appliance.

4.      Extreme caution is necessary when any appliance is used by or near children and whenever the appliance is left operating and unattended.

5.      This appliance is not intended for use in wet locations. Never locate appliance where it may fall into a water container.

Do not insert or allow foreign objects to enter any ventilation or exhaust opening as this may cause an electrical shock, fire or damage to the appliance.

Benchmark USA Inc.  ●  25-J Brookfield Oaks Dr. ●  Greenville, SC 29607 ●  T: 864-312-5500  ●  F: 864-312-5505

OPERATING INSTRUCTIONS

- Apply the decorative POPCORN cornice to the top of the machines using two pan-head screws.

- Plug the machine into a properly grounded 120 volt power supply.

- Install a 50 watt or smaller light bulb in the socket in the chassis (not necessary for operation).  For best results use a 50 watt R20 mini flood bulb.

- Turn on the “MASTER” switch and the interior lamp will come on and the warming element in the base will be activated.

- Turn on the “KETTLE” switch and the heat to the kettle and the agitating motor will be activated.  Wait approximately 5 minutes to allow the kettle to get up to operating temperature.

- Place ½ cup of raw popcorn kernels, 1/6 cup of cooking oil and 1 teaspoon of seasoning salt into the kettle and lower the kettle lid.  (use Benchmark USA part # 42004 – Popcorn measure kit)

- Allow the corn to pop until the popping ceases (3 seconds between pops) then dump the kettle by turning the handle clock-wise and pulling downward.

- For additional batches repeat steps 5 & 6.  It will not be necessary to wait 5 minutes since the kettle will already be up to operating temperature.

- After popping the final batch, turn off the “KETTLE” switch and with a damp cloth wipe any excess oil from the inside and outside of the kettle to prevent burned on oil stains.  Use caution as the kettle will be HOT!

- If you leave the “MASTER” switch in the on position, the warming element in the base will keep any popped corn fresh and warm.  The interior lamp will remain on as well and serve as a reminder that the popper is on.

KETTLE CARE

CAUTION:  NEVER SUBMERGE THE KETTLE IN ANY LIQUID!  IT WILL SHORT OUT THE ELECTRONIC COMPONENTS INSIDE OF IT AND VOID THE WARRANTY!

- Always turn off the “KETTLE” switch after popping the final batch of a session.  Leaving it on will burn any residue oil that is left in the kettle.

- For routine cleaning: After popping the final batch of a session, use a damp cloth to wipe the excess oil from the interior and exterior of the kettle.

- Benchmark USA kettles are anodized to create an easy cleaning, non-stick surface.  However, if excess oil is not removed it will eventually carbonize on the surface.  For heavily built-up carbon removal it is recommended to use (Benchmark USA part # 43001 – kettle cleaning kit).

CABINET CARE

Always unplug the machine from the power supply before cleaning!

- The Stainless steel bottom tray, old-maid drawer and drop door are removable for easy cleaning in a sink or dishwasher.  Use a mild detergent and water.

- Use a standard glass cleaner for the glass panels and corner posts.

Page 2

Page 3

Page 4

4 OUNCE KETTLE ASSEMBLY

(complete parts list on page 7)

Page 5

CHASSIS ASSEMBLY

(complete parts list on page 7)

Page 6

PARTS LIST

| P/N | Description |  | P/N | Description |
| --- | --- | --- | --- | --- |
| 0014 | 4 ounce kettle -120 volt |  | 4042 | Left door |
| 0024 | 4 ounce kettle -240 volt |  | 4041 | Right door |
| 0016 | 6 ounce kettle -120 volt |  | 4012 | Door handle |
| 0026 | 6 ounce kettle -240 volt |  | 4015 | Door magnet |
| 0018 | 8 ounce kettle -120 volt |  | 4020 | Door hinge set |
| 0028 | 8 ounce kettle -240 volt |  | 4033 | Base - complete |
| 1001 | Popper Assembly |  | 4035 | Terminal box cover |
| 1002 | Chassis Assembly |  | 5001 | Left kettle hanger |
| 1004 | Wiring Diagram |  | 5002 | Right kettle hanger assembly |
| 1011 | 50w Warmer element -120 volt |  | 5003 | Lamp Guard |
| 1012 | Warmer clamp |  | 5004 | 4 ounce kettle lid |
| 1021 | 50w warmer element -240 volt |  | 5006 | 6 ounce kettle lid |
| 1014 | 800w kettle element -120 volt |  | 5008 | 8 ounce kettle lid |
| 1024 | 800w kettle element -240 volt |  | 5009 | Kettle latch |
| 1016 | 1000w kettle element -120 volt |  | 5014 | 4 ounce kettle stirrer |
| 1026 | 1000w kettle element -240 volt |  | 5016 | 6 ounce kettle stirrer |
| 1018 | 1250w kettle element -120 volt |  | 5018 | 8 ounce kettle stirrer |
| 1028 | 1250w kettle element -240 volt |  | 5021 | Kettle handle complete |
| 1039 | Thermostat |  | 5022 | Kettle handle spring |
| 2001 | Power switch |  | 5023 | Kettle hinge-pin/clip |
| 2002 | Kettle switch |  | 5025 | Kettle lid magnet |
| 2005 | Lamp socket |  | 5027 | Corner Post |
| 2006 | Kettle socket |  | 5029 | Heater Clamp |
| 2007 | Kettle plug |  | 5031 | Chassis |
| 2008 | Kettle pigtail w/plug |  | 5034 | 4 oz. Kettle jacket |
| 2009 | Power cord w/plug |  | 5036 | 6 oz. Kettle jacket |
| 2018 | 120 volt (50/60 hz) gear motor |  | 5038 | 8 oz. Kettle jacket |
| 2028 | 240 volt (50/60 hz) gear motor |  | 5044 | 4 oz. kettle inner |
| 4021 | Side glass |  | 5046 | 6 oz. kettle inner |
| 4022 | Front glass |  | 5048 | 8 oz. kettle inner |
| 4003 | Glass channel set |  |  |  |
| 4006 | Drawer |  |  |  |
| 4023 | Decorative cornice metro |  |  |  |
| 4024 | Black canopy top |  |  |  |
| 4027 | Stainless product tray |  |  |  |
| 4028 | Stainless drop door |  |  |  |

To Order Parts Contact:

Benchmark USA, Inc.

25-J Brookfield Oaks Dr.

Greenville, SC 29607 USA

Tel:  877-432-5500

Fax:  864-312-5505

Email:  parts@benchmarkusainc.com

Page 7

Three Year Warranty

THE WARRANTY:

Benchmark USA warrants the original purchaser of new food equipment manufactured by Benchmark USA to be free from defects in material or workmanship  from the date of the original purchase for a period of three years.

WHO IS COVERED BY THIS WARRANTY:

The original purchaser who has provided proper proof of the original retail purchase and all other requested information.

WHAT IS WARRANTIED:

The obligation of Benchmark USA under this warranty is limited to the replacement of any parts or components that in the opinion of Benchmark USA are defective, F.O.B. the factory, or at any other location that Benchmark USA may designate.

WHAT VOIDS THE WARRANTY:

1.  Any piece of equipment that has been installed, operated or maintained inconsistently

with Benchmark USA operating instructions.

2.  Any  part  or  component  that has been modified, changed or altered from its original

configuration.

3.  Any   electrical  component   that   has  been   damaged  due  to  submergence  in  any

liquid.

4.  Any   part   or   component   that   has   been   subject   to  abuse,  misuse,  neglect  or

accidents.

EXTENT OF WARRANTY:

Any defective component, part or assembly returned to Benchmark USA will be replaced by Benchmark USA.  Benchmark USA will not be responsible for any expenses incurred by the customer under the terms of this warranty, nor shall it be responsible for any damages either consequential, special, contingent, or otherwise; or expenses or injury arising directly or indirectly from the use of a Benchmark USA product.  Any component, part or assembly returned to Benchmark USA must be returned at the customers expense along with proof of purchase.  Benchmark USA reserves the right to determine whether the terms of this warranty have been properly complied with.  In the event that the terms are not complied with, Benchmark USA shall be under no obligation to honor this warranty.

Benchmark USA Inc.  ●  25-J Brookfield Oaks Dr. ●  Greenville, SC 29607 ●  T: 864-312-5500  ●  F: 864-312-5505